PROOF OF CONCEPT GUIDE

Secure CAN-FD Firmware Update over UDS (ISO-TP)

nRF52 + Zephyr + MCUboot POC — Migration Path to i.MX RT1176 EV-Bike Cluster

Document Version: 0.1 (Draft)

Date: 28 July 2026

Related Project: EV-Bike Instrument Cluster — FOTA (FR-01, FR-15)

# Table of Contents

# 1. POC Objective

Demonstrate a secure CAN-FD firmware update over UDS (ISO-TP) into a Zephyr-based MCU using MCUboot, with rollback capability.

## 1.1 Questions the POC Must Answer

✓  Can we transfer a firmware image over CAN-FD?

✓  Can ISO-TP transport large firmware images?

✓  Can UDS control the flashing process?

✓  Can the target write images safely into flash?

✓  Can MCUboot validate and boot the new firmware?

If these are proven, the transport architecture is validated.

# 2. Rationale — Why Not Start Directly on the RT1176

This is recommended as a first POC ahead of RT1176 bring-up. Using a simpler master/target pairing is easier to debug and validates roughly 90% of the complete FOTA architecture before committing to production hardware.

| Role | Recommended Hardware |
| --- | --- |
| Master / Tester | BeagleBone Black (Linux) or STM32 (Zephyr or bare-metal) |
| Target ECU | nRF52 (Zephyr + MCUboot) |

Once the transport and bootloader flow is proven on nRF52, porting the target ECU to the RT1176 is primarily a BSP and flash-driver exercise — provided the software above the CAN driver layer is written against generic Zephyr APIs (see Section 12).

# 3. Overall POC Architecture

HOST / TESTER BBB Linux OR STM32 Master -------------------------------- UDS Client ISO-TP Client CAN-FD Driver Flash Tool ¦ ¦ CAN-FD Physical Bus ¦ v ============================================ Target ECU (nRF52) Zephyr Application ¦ -------------------------- UDS Server ISO-TP CAN Driver FOTA Manager Flash Manager MCUboot APIs -------------------------- ¦ Internal Flash ¦ Secondary Slot ¦ MCUboot Bootloader ¦ Primary Application

# 4. POC Deliverables

| Deliverable | Status |
| --- | --- |
| CAN-FD communication | ✓ |
| ISO-TP transport | ✓ |
| UDS download | ✓ |
| Image storage | ✓ |
| MCUboot image validation | ✓ |
| Automatic reboot | ✓ |
| Rollback | ✓ |
| Flash interruption recovery | ✓ |

# 5. Suggested Team Split

Rather than one engineer owning the full stack, split responsibility across three tracks:

| Engineer | Track | Responsible For |
| --- | --- | --- |
| Engineer 1 | Transport | CAN driver, ISO-TP, UDS |
| Engineer 2 | Bootloader | Flash layout, MCUboot, image manager, rollback |
| Engineer 3 | Tester Tool | Linux / Python UDS client, firmware sender, automation |

# 6. Phase-by-Phase Plan

## Phase 1 — CAN-FD Bring-Up

Goal: verify CAN communication only — no ISO-TP, no UDS, no flash.

Tester --Transmit CAN FD Frame--> Target --Receive--> Print UART Example: CAN ID = 0x700 Payload = 01 02 03 04 Target UART: "Received CAN ID = 0x700, Length = 4"

### Success Criteria

- 1 Mbps arbitration, 2–5 Mbps data phase

- 64-byte payload

- Interrupt-driven RX and TX

- No bus-off events

## Phase 2 — ISO-TP

Goal: transport firmware-sized packets. Firmware stays in RAM — no flash writes at this stage.

Firmware.bin -> ISO-TP -> CAN-FD Frames -> Target -> Reassemble Example: 120 KB firmware -> Frame1, Frame2, Frame3, ... FrameN Target: Receive -> Reassemble -> CRC -> OK

### Success Criteria

- Receive 128 KB without corruption

## Phase 3 — Minimal UDS

Implement only the services needed for a download flow — ignore the remaining 50+ UDS services.

| Service ID | Description |
| --- | --- |
| 0x10 | Diagnostic Session Control |
| 0x11 | ECU Reset |
| 0x22 | Read Data by Identifier (DID) |
| 0x27 | Security Access |
| 0x34 | Request Download |
| 0x36 | Transfer Data |
| 0x37 | Transfer Exit |

Tester -> 0x10 -> Target -> Positive Response Tester -> 0x27 -> Target -> Seed -> Key -> Unlocked Tester -> 0x34 -> Target -> Download OK Tester -> 0x36 -> Target -> ... Tester -> 0x37 -> Target -> Complete

## Phase 4 — Flash Manager

Introduce flash writes.

Transfer Data -> FOTA Manager -> Flash API -> Secondary Slot

### Responsibilities

- Erase, program, verify, CRC

- Never write into the running (primary) image

- Always write into the secondary slot

### Success Criteria

- Image stored 100%, CRC OK

## Phase 5 — MCUboot Integration

Secondary Slot -> Image Header -> Signature -> MCUboot -> Validation -> Swap -> Boot

### Flow

Download -> Verify -> Image OK -> Reset -> MCUboot -> Swap -> Run

# 7. Flash Layout (POC)

+----------------------------------+ ¦ Boot ROM ¦ +----------------------------------+ ¦ MCUboot ¦ +----------------------------------+ ¦ Primary Slot (Current App) ¦ +----------------------------------+ ¦ Secondary Slot (New App) ¦ +----------------------------------+ ¦ Scratch (if swap strategy needs)¦ +----------------------------------+ ¦ Settings ¦ +----------------------------------+

# 8. Firmware Update State Machine

Idle ¦ v Diagnostic Session ¦ v Security Access ¦ v Request Download ¦ v Receive Blocks ¦ v Verify CRC --(PASS/FAIL)--> ¦ v Image Ready ¦ v ECU Reset ¦ v MCUboot ¦ v New Firmware

# 9. Failure Recovery Test Plan

The POC should intentionally exercise failure paths, not just the happy path.

| Test | Condition | Expected Result |
| --- | --- | --- |
| Test 1 | Disconnect CAN halfway through transfer | Download resumes, or aborts safely |
| Test 2 | Power OFF during flashing | Old firmware still boots |
| Test 3 | Corrupted image | MCUboot rejects the image |
| Test 4 | Wrong signature | Authentication fails |
| Test 5 | Application crashes after update | Rollback to previous image |

# 10. Suggested Repository Structure

canfd-fota-poc/ ¦ ¦-- host/ ¦ ¦-- uds_client/ ¦ ¦-- isotp/ ¦ ¦-- firmware_sender/ ¦ `-- scripts/ ¦ ¦-- target/ ¦ ¦-- zephyr_app/ ¦ ¦-- can/ ¦ ¦-- isotp/ ¦ ¦-- uds_server/ ¦ ¦-- flash_manager/ ¦ ¦-- mcuboot/ ¦ `-- config/ ¦ ¦-- docs/ ¦ ¦-- architecture.md ¦ ¦-- sequence_diagrams.md ¦ ¦-- flash_layout.md ¦ ¦-- test_plan.md ¦ `-- troubleshooting.md ¦ `-- tools/

# 11. Suggested Timeline

| Week | Goal |
| --- | --- |
| 1 | Bring up CAN-FD communication and verify reliable frame transfer. |
| 1 | Implement ISO-TP segmentation/reassembly and validate large payload transfers (100–500 KB). |
| 1 | Add the minimal UDS services (0x10, 0x11, 0x22, 0x27, 0x34, 0x36, 0x37). |
| 2 | Implement flash writing to the secondary slot, integrate MCUboot image validation, and complete reboot/rollback testing. |

# 12. Recommendation Specific to the RT1176 EV-Bike Project

The end goal is an i.MX RT1176 + Zephyr + CAN-FD FOTA platform for the EV-bike cluster. To maximize code reuse from this POC, keep the software hardware-independent above the CAN driver layer:

Application ¦ UDS Download Manager ¦ ISO-TP Transport ¦ CAN Abstraction API ¦ +------------+------------+ ¦ ¦ nRF52 BSP RT1176 BSP

If the UDS server, ISO-TP engine, flash manager, and FOTA state machine are written against generic Zephyr APIs (drivers/can, flash_map, stream_flash, and MCUboot interfaces), migrating from the nRF52 POC to the RT1176 EVK should require mainly:

- Replacing the CAN driver binding

- Updating the flash partition layout

- Adjusting the devicetree

- Validating the RT1176-specific memory/boot configuration

This approach maximizes code reuse and minimizes platform-specific rework when transitioning from the proof of concept to the production EV-bike platform.